House Collection Robot

- Drive straight until past edge of starting box (Measured distance)

- Turn left 90 degrees (Measured turn)

- Drive forward and turn to align with row of houses across the board. (Measured distance and measured turn)

- Reverse until both touch sensors are triggered, ensuring the robot is properly lined up.

- Drive straight across the board, collecting the houses. (Measured distance)

- Lower the arm with “shields” onto houses.

- Turn 180 degrees (Measured turn)

- Drive straight until past the starting box, by the “ocean” (Measured distance)

- Turn right 90 degrees (Measured turn)

- Drive straight until within the horizontal projection of the starting box. (Measured distance)

- Turn right 90 degrees (Measured turn)

- Drive straight into starting box. (Measured distance)

- Verify that the robot is in the starting box, using bump sensors to move all the way to the back of the box.

- Cut power when round ends.

- The (Measured distance) signifies that we wrote a method that uses the motor ticks to travel an exact distance, and we will use this to move precisely where we need to go.

- The (Measured turn) signifies we have a similar method for turning. Parameters are a number of degrees, meaning we can make exact turns.