# Rocket Flight Check List

Run through this list of tasks before launching…

A. Cargo to Engine Base

- Check separation between cargo and engine base.  It should be easy to separate but not fall off.  Use baby powder to help.

- Insert engine, use tape to make it snug, and tape it on the tube.

- Fold parachute in the “z” format, wrap shrouds loosely around chute.

- Insert wadding or chute protector.

- Insert parachute, shock cord loose, use baby powder to keep it loose.

- Close up cargo and engine base.

B. Cargo Setup

- Wrap egg in plastic wrap.

- Insert egg with foam protection.

- Put battery in altimeter, tape the battery in, and immediately insert in cargo. You have 25 seconds to do this. Do not let the sun shine on the altimeter.

- Insert altimeter with beeper down, be careful not to trip the sensor, be sure to have access to air hole.

- Place nose cone on cargo, insert locking pins, and tape to cargo.

- Listen for altimeter ready beeping.  If tripped, go back to number 3.

- Weigh the rocket and record the grams.

C. Load on Launch Tower

- Insert igniter and plug (black for E9s), fold out wires

- WD40 and steel wool launch rod.

- Load on tower, be sure it is loose and can move up tower.

- Connect launch tower line clips

- One last look

- Connect launch control to launch tower line.

D. Launch

- Clear launch area

- Send out recovery team

- Check if photographers ready

- Check if timers ready

- Tell range safety officer OK for launch

E. Recovery

- Note recovery location – compass direction and distance

- Record time and other flight data

- Take rocket to officials for evaluation

- Remove egg and altimeter

- Listen for altitude

- Plug altimeter into computer and download data