# Rocket Flight Check List

Run through this list of tasks before launching…

A. Cargo to Engine Base

- Check separation between cargo and engine base.  It should be easy to separate but not fall off.  Use baby powder to help.

- Insert engine, make sure it is clipped in.

- Baby powder the chute, fold parachute in the “z” format, wrap shrouds loosely around chute, but never cross the shroud wrappings.

- Insert wadding and chute protector.

- Insert parachute, shock cord loose, use baby powder to keep it loose.

- Close up cargo and engine base.

B. Cargo Setup

- Put egg in plastic bag.

- Insert egg in nose cone with foam protection.

- Put battery in altimeter, insert into cargo unit, hold tight with wires and baffle. You have 20 seconds to do this. Do not let the sun shine on the altimeter.

- Be careful not to trip the sensor, be sure it has access to air holes.

- Place nose cone on cargo, insert locking pins, and tape to cargo.

- Listen for altimeter in ready beeping state.  If tripped, go back to number 3.

- Weigh the rocket and record the grams.

C. Load on Launch Tower

- WD40 and steel wool launch rod.

- Insert igniter and plug (black for E9s), fold out wires.

- Load on tower, be sure it is loose and can move up tower.

- Connect launch tower line clips.

- One last look.  Ask a buddy to check.

- Connect launch control to launch tower line.

D. Launch

- Clear launch area.

- Send out recovery team.

- Check if photographers ready.

- Check if timers ready.

- Tell range safety officer OK for launch.

E. Recovery

- Note recovery location – compass direction and distance.

- Record time and other flight data.

- Take rocket to officials for evaluation.

- Remove egg and altimeter.

- Listen for altitude.

- Plug altimeter into computer and download data.