# Rocket Flight Check List

A. Cargo to Motor Base

- Check separation between cargo and motor base (booster).  It should be easy to separate but not fall off.  Use baby powder to help.

- Insert motor, make sure it is clipped in, don't forget the spacer, be careful with igniter.

- Put a little 'dog barf' in the top of the motor mount, on top of the motor ejection charge, use a stick to insert.

- Baby powder the chutes, fold parachute in the “z” format, wrap shrouds loosely around chute, but never cross the shroud wrappings.

- Insert chute protector over the motor mount.

- Insert chutes, shock cords loosely; use baby powder to keep it loose.

- Close up cargo and motor base.

B. Cargo Setup

- Put eggs in plastic bag. Insert eggs in nose cone with foam protection.

- Put battery in altimeter, insert into cargo unit holder with black pressure sensor up, hold tight with wires. Insert holder into cargo unit, use foam pads to keep holder tight. You have 20 seconds to do this. Do not let the sun shine on the altimeter.

- Be careful not to trip the altimeter sensor, be sure it has access to air holes.

- Place nose cone on cargo, twist to lock.  Black marks should be aligned.

- Shake rocket lightly to be sure nothing rattles around and cargo (eggs) are secured.

- Listen for altimeter in ready beeping state.  If tripped, go back to number 3.

- Check in, weigh the rocket and record the grams.

C. Load on Launch Tower

- WD40 and steel wool launch rod.

- Load on tower, be sure it is loose and can move up tower.

- Angle launch rod with the wind, one degree for every mile per hour of wind.

- Use a plum bob to check the angle and direction.

- Connect launch power line clips.

- One last look.  Ask a buddy to check.

- If you think you have a problem or forgot something - Stop Now.

D. Launch

- Clear launch area. Send out recovery team.

- Check if photographers ready. Check if timers ready.

- Tell range safety officer OK for launch, signal ready to launch.

E. Recovery

- Recovery team needs to walk/run to landing area before it lands. Don't catch.

- Carefully pick up cargo unit - do not remove eggs or altimeter. Just listen.

- Don't forget the booster.

- Take rocket (everything) to officials for evaluation.

- Remove egg and altimeter with officials watching.

- Plug altimeter into computer and download data.